Table 1 — Register & Flag Summary

| Register / Flag | Size (bits) | Purpose |
| --- | --- | --- |
| R0 – R7 | 16 bits each | General-purpose registers. Used as source and destination for all ALU, load, store, and branch instructions. |
| PC | 12 bits | Program Counter. Holds the address of the next instruction to fetch. |
| IR | 16 bits | Instruction Register. Holds the currently executing instruction after fetch. |
| TR | 16 bits | Temporary Register. Used internally during interrupt cycle and SWAP to save/restore values. |
| AR | 12 bits | Address Register. Holds the memory address for the current memory access. |
| DR | 16 bits | Data Register. Holds data read from memory during a load operation. |
| INPR | 8 bits | Input Register. Receives 8-bit data from the input device. |
| OUTR | 8 bits | Output Register. Sends 8-bit data to the output device. |
| Z Flag | 1 bit | Zero Flag. Set to 1 when the ALU result is zero. |
| C Flag | 1 bit | Carry Flag. Set to 1 when an arithmetic operation produces a carry-out or a shift shifts out a bit. |
| N Flag | 1 bit | Negative Flag. Set to 1 when the ALU result has bit 15 = 1 (negative in two's complement). |
| IEN | 1 bit | Interrupt Enable flip-flop. When 1, interrupts are allowed. |
| FGI | 1 bit | Input Flag. Set to 1 when the input device has data ready in INPR. |
| FGO | 1 bit | Output Flag. Set to 1 when the output device is ready to accept data from OUTR. |
| S | 1 bit | Start-Stop flip-flop. When 0, the CPU halts execution. |
| SC | 4 bits | Sequence Counter. Counts timing pulses T0–T5 to sequence microoperations. |

Table 2 — Instruction Format Fields

| Field | Bit Positions | Size | Purpose |
| --- | --- | --- | --- |
| OPCODE | 15 – 11 | 5 bits | Identifies the instruction. 5 bits allow up to 32 unique opcodes. |
| MODE | 10 – 9 | 2 bits | Addressing mode. 00 = Register, 01 = Immediate, 10 = Direct, 11 = Register-Indirect. |
| DST | 8 – 6 | 3 bits | Destination register selector. 3 bits encode R0–R7. |
| SRC | 5 – 0 | 6 bits | Source field. In register mode: bits [2:0] select Rs. In immediate mode: 6-bit literal. In jump instructions: 9-bit address uses DST+SRC combined. |

Table 3 — ALU Control Signal Table

| S3 | S2 | S1 | S0 | Cin | Operation | Function | Instruction |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | F = A + B | Add | ADD |
| 0 | 0 | 0 | 1 | 1 | F = A + B' + 1 | Subtract (2's complement) | SUB, CMP |
| 0 | 0 | 1 | 0 | 0 | F = A + 1 | Increment | INC |
| 0 | 0 | 1 | 1 | 1 | F = A − 1 | Decrement | DEC |
| 0 | 1 | 0 | 0 | 0 | F = A × B (low 16 bits) | Multiply | MUL |
| 0 | 1 | 0 | 1 | — | F = A << n | Shift Left logical | SHL |
| 0 | 1 | 1 | 0 | — | F = A >> n (zero-fill) | Shift Right logical | SHR |
| 0 | 1 | 1 | 1 | — | F = B (pass-through) | Transfer B | MOV, LDI, LDM |
| 1 | 0 | 0 | 0 | — | F = A AND B | Bitwise AND | AND |
| 1 | 0 | 0 | 1 | — | F = A OR B | Bitwise OR | OR |
| 1 | 0 | 1 | 0 | — | F = A XOR B | Bitwise XOR | XOR |
| 1 | 0 | 1 | 1 | — | F = NOT A | Bitwise Complement | NOT, CMR |
| 1 | 1 | 0 | 0 | — | F = 0 | Force zero | CLA |
| 1 | 1 | 0 | 1 | — | Reserved | — | — |
| 1 | 1 | 1 | 0 | — | Reserved | — | — |
| 1 | 1 | 1 | 1 | — | Reserved | — | — |

Table 4 — Register-Reference One-Hot Encoding

| Mnemonic | Trigger Bit (IR[8:0]) | Microoperations | Description |
| --- | --- | --- | --- |
| CLA | IR[8] = 1 | T3: R0 ← 0, SC ← 0 | Clear R0 to zero. |
| CMR Rd | IR[7] = 1 | T3: A ← RF[DST] T4: RF[DST] ← NOT A set Z/N flags, SC ← 0 | Complement any destination register. |
| CLC | IR[6] = 1 | T3: C ← 0, SC ← 0 | Clear Carry flag. |
| STC | IR[5] = 1 | T3: C ← 1, SC ← 0 | Set Carry flag to 1. |
| CLZ | IR[4] = 1 | T3: Z ← 0, SC ← 0 | Clear Zero flag. |
| NOP | IR[3] = 1 | T3: SC ← 0 | No operation. Immediately resets sequence counter. |
| SWAP Rd, Rs | IR[2] = 1 | T3: TR ← RF[DST] T4: RF[DST] ← RF[SRC] T5: RF[SRC] ← TR, SC ← 0 | Swap two registers using TR as temp. Only reg-ref instruction needing 3 cycles. |

Table 5 — Master Microoperation Summary

| Instruction | Category | Microoperation (T-Notation) |
| --- | --- | --- |
| Fetch / Decode | Fetch / Decode | T0: AR ← PC T1: IR ← M[AR], PC ← PC + 1 T2: OPCODE ← IR[15:11], MODE ← IR[10:9], DST ← IR[8:6], SRC ← IR[5:0] |
| ADD / SUB / MUL / AND / OR / XOR / MOV | Register-to-Register (MODE=00) | T3: A ← RF[DST], B ← RF[SRC] T4: RF[DST] ← ALU_result, set Z/C/N flags, SC ← 0 |
| LDI Rd, #imm | Load Immediate | T3: RF[DST] ← IR[5:0] (6-bit immediate), SC ← 0 |
| LDM Rd, [Rs] | Load from Memory | T3: AR ← RF[SRC] T4: DR ← M[AR] T5: RF[DST] ← DR, SC ← 0 |
| STM [Rd], Rs | Store to Memory | T3: AR ← RF[DST] T4: M[AR] ← RF[SRC], SC ← 0 |
| JMP addr | Jump Unconditional | T3: PC ← IR[8:0], SC ← 0 |
| JZ Rd, addr | Jump if Zero | T3: if (RF[DST] = 0) then PC ← IR[5:0]; SC ← 0 |
| JNZ Rd, addr | Jump if Not Zero | T3: if (RF[DST] ≠ 0) then PC ← IR[5:0]; SC ← 0 |
| NOT / INC / DEC Rd | Single-Register | T3: A ← RF[DST] T4: RF[DST] ← ALU_result (NOT A / A+1 / A−1), SC ← 0 |
| SHL / SHR Rd, #n | Shift | T3: A ← RF[DST], n ← IR[3:0] T4: RF[DST] ← A shifted by n, set C if bits shifted out, SC ← 0 |
| CMP Rd, Rs | Compare | T3: A ← RF[DST], B ← RF[SRC] T4: temp ← A − B, set Z/C/N flags only (no write-back), SC ← 0 |
| CMP Rd, Rs | Compare |  |
| HLT | Halt | T3: S ← 0, SC ← 0 |
| — (Interrupt Cycle) | Interrupt | Condition: T0.T1.T2.IEN.(FGI+FGO): R ← 1 RT0: AR ← 0, TR ← PC RT1: M[AR] ← TR, PC ← 0 RT2: PC ← PC + 1, IEN ← 0, R ← 0, SC ← 0 |
| INP | Input | T3: R0 ← INPR, FGI ← 0, SC ← 0 |
| OUT | Output | T3: OUTR ← R0[7:0], FGO ← 0, SC ← 0 |
| SKI | Skip if Input Ready | T3: if FGI = 1: PC ← PC + 1; SC ← 0 |
| SKO | Skip if Output Ready | T3: if FGO = 1: PC ← PC + 1; SC ← 0 |
| ION | Interrupt Enable | T3: IEN ← 1, SC ← 0 |
| IOF | Interrupt Disable | T3: IEN ← 0, SC ← 0 |
| CLA (IR[8]=1) | Reg-Ref Group | T3: R0 ← 0, SC ← 0 |
| CMR (IR[7]=1) | Reg-Ref Group | T3: A ← RF[DST] |
| CMR (IR[7]=1) | Reg-Ref Group | T3: RF[DST] ← NOT A, set Z/N, SC ← 0 |
| CLC (IR[6]=1) | Reg-Ref Group | T3: C ← 0, SC ← 0 |
| STC (IR[5]=1) | Reg-Ref Group | T3: C ← 1, SC ← 0 |
| CLZ (IR[4]=1) | Reg-Ref Group | T3: Z ← 0, SC ← 0 |
| NOP (IR[3]=1) | Reg-Ref Group | T3: SC ← 0 |
| SWAP (IR[2]=1) | Reg-Ref Group | T3: TR ← RF[DST] T4: RF[DST] ← RF[SRC] T5: RF[SRC] ← TR, SC ← 0 |