Tables:

Fetch–Decode–Execute control signal timeline

| Timing | Active signals asserted by CU | What happens |
| --- | --- | --- |
| T0 | AR_Load, PC_to_AR_bus | AR ← PC (address register gets current PC) |
| T1 | MemRead, IR_Load, PC_Inc | IR ← M[AR]; PC ← PC+1 simultaneously |
| T2 | Decoder_Enable, SC_Separator | Opcode decoded, D0–D31 stabilize; MODE/DST/SRC fields latched |
| T3+ | Instruction-specific signals from PLA | Execute stage — varies per opcode as defined in Step 5 |
| SC=0 | SC_CLR pulse | Sequence counter resets → next cycle begins at T0 |

ALU Control Signal Table

5 control bits total: S2 S1 S0 select the operation group; Cin is the carry input. ALU selects between arithmetic unit, logic unit, and shifter via an internal 4-to-1 MUX.

| S2 | S1 | S0 | Cin | Function | Expression | Type | Opcode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Arithmetic operations (S2 = 0, routes through adder) |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | Transfer A | F = A | Arith | MOV |
| 0 | 0 | 0 | 1 | Increment | F = A + 1 | Arith | INC |
| 0 | 0 | 1 | 0 | Add | F = A + B | Arith | ADD |
| 0 | 0 | 1 | 1 | Add with carry | F = A + B + 1 | Arith | — |
| 0 | 1 | 0 | 0 | Add complement | F = A + B̄ | Arith | — |
| 0 | 1 | 0 | 1 | Subtract | F = A − B (2's comp) | Arith | SUB / CMP |
| 0 | 1 | 1 | 0 | Decrement | F = A − 1 | Arith | DEC |
| 0 | 1 | 1 | 1 | Transfer A | F = A (no change) | Arith | — |
| Logic operations (S2 = 1, S1 = 0 → routes through logic unit) |  |  |  |  |  |  |  |
| 1 | 0 | 0 | x | AND | F = A AND B | Logic | AND |
| 1 | 0 | 1 | x | OR | F = A OR B | Logic | OR |
| 1 | 0 | 0 | x | XOR | F = A XOR B | Logic | XOR |
| 1 | 0 | 1 | x | NOT | F = NOT A (B unused) | Logic | NOT / CMR |
| Shift operations (S2 = 1, S1 = 1 → routes through barrel shifter) |  |  |  |  |  |  |  |
| 1 | 1 | 0 | x | Shift left logical | F = A << n (n from IR[3:0]) | Shift | SHL |
| 1 | 1 | 1 | x | Shift right logical | F = A >> n (zero-fill) | Shift | SHR |

Step 5 Complete: Microoperations for ALL Remaining Instructions

Here are the missing microops in full T-notation, ready for Section 10 of the report.

5.14 INP (Input)

T3: If FGI = 1: R0 ← INPR, FGI ← 0, SC ← 0

If FGI = 0: SC ← 0 (no-op — use SKI to poll before INP)

5.15 OUT (Output)

T3: If FGO = 1: OUTR ← R0[7:0], FGO ← 0, SC ← 0

If FGO = 0: SC ← 0 (no-op — use SKO to poll before OUT)

5.16 SKI (Skip if Input Ready)

T3: If FGI = 1: PC ← PC + 1

SC ← 0

5.17 SKO (Skip if Output Ready)

T3: If FGO = 1: PC ← PC + 1

SC ← 0

5.18 ION (Interrupt Enable)

T3: IEN ← 1, SC ← 0

5.19 IOF (Interrupt Disable)

T3: IEN ← 0, SC ← 0

5.20 Register-Reference Group (opcode = 11110, decoded by one-hot bits [8:2])

The control unit checks IR[15:11] = 11110, then decodes the one-hot function bits:

CLA  (IR[8] = 1):   T3: R0 ← 0, SC ← 0

CMR  (IR[7] = 1):   T3: A ← RF[DST]

T4: RF[DST] ← NOT A, set Z/N flags, SC ← 0

CLC  (IR[6] = 1):   T3: C ← 0, SC ← 0

STC  (IR[5] = 1):   T3: C ← 1, SC ← 0

CLZ  (IR[4] = 1):   T3: Z ← 0, SC ← 0

NOP  (IR[3] = 1):   T3: SC ← 0  (no operation — immediately resets)

SWAP (IR[2] = 1):   T3: TR ← RF[DST]

T4: RF[DST] ← RF[SRC]

T5: RF[SRC] ← TR, SC ← 0

Note for report: SWAP uses the TR (Temp Register) as an intermediary, exactly as Mano's Machine uses TR for certain transfer operations. This is the only register-reference instruction that needs 3 execution cycles.

6.2 ALU Control Signal Table

| S3 | S2 | S1 | S0 | Cin | Function | Instruction |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | F = A + B | ADD |
| 0 | 0 | 0 | 1 | 1 | F = A + B̄ + 1 (2's comp) | SUB, CMP |
| 0 | 0 | 1 | 0 | 0 | F = A + 1 | INC |
| 0 | 0 | 1 | 1 | 1 | F = A − 1 (A + 1̄ + 1) | DEC |
| 0 | 1 | 0 | 0 | 0 | F = A × B (low 16) | MUL |
| 0 | 1 | 0 | 1 | — | F = A SHL n | SHL |
| 0 | 1 | 1 | 0 | — | F = A SHR n (logical) | SHR |
| 0 | 1 | 1 | 1 | — | F = B (pass-through) | MOV, LDI, LDM |
| 1 | 0 | 0 | 0 | — | F = A AND B | AND |
| 1 | 0 | 0 | 1 | — | F = A OR B | OR |
| 1 | 0 | 1 | 0 | — | F = A XOR B | XOR |
| 1 | 0 | 1 | 1 | — | F = NOT A | NOT, CMR |
| 1 | 1 | 0 | 0 | — | F = 0 (force zero) | CLA |
| 1 | 1 | 0 | 1 | — | Reserved | — |
| 1 | 1 | 1 | 0 | — | Reserved | — |
| 1 | 1 | 1 | 1 | — | Reserved | — |

Step 5 — Microoperations (remaining instructions)

I/O group (6 instructions) + Register-Reference group (7 instructions) — T-notation for report Section 10

Fetch & Decode (T0–T2) are the same for ALL instructions and already defined. Every execution sequence below starts at T3 and ends with SC ← 0.

5A — I/O Instructions

INPI/ORead character from input device into R0

T3:R0 ← INPR, FGI ← 0, SC ← 0 — copy 8-bit INPR to R0, clear input flag

OUTI/OSend lower 8 bits of R0 to output device

T3:OUTR ← R0[7:0], FGO ← 0, SC ← 0 — latch R0 low byte, clear output flag

SKII/OSkip next instruction if input flag (FGI) = 1

T3:If (FGI = 1): PC ← PC + 1 — input device ready → skip the busy-wait

SC ← 0

SKOI/OSkip next instruction if output flag (FGO) = 1

T3:If (FGO = 1): PC ← PC + 1 — output device ready → skip the busy-wait

SC ← 0

IONI/OEnable interrupt system

T3:IEN ← 1, SC ← 0 — turn interrupt enable flip-flop on

IOFI/ODisable interrupt system

T3:IEN ← 0, SC ← 0 — turn interrupt enable flip-flop off

SKI/SKO polling loop pattern: the program loops on SKI/SKO until the flag goes high, then falls through to INP/OUT. This is exactly the program-controlled I/O mechanism from Lecture 21.

5B — Register-Reference Instructions (opcode 11110, one-hot bits [8:0])

All reg-ref instructions share opcode 11110. After T0–T2 (fetch/decode), the Control Unit checks which bit of IR[8:0] is set and activates the corresponding microoperation. Only one bit is 1 at a time (one-hot encoding).

CLAReg-RefClear R0 (bit 8 of IR[8:0])

T3:If (IR[8] = 1): R0 ← 0, SC ← 0 — zero out R0, set flags Z=1 C=0 N=0

CMR RdReg-RefComplement register Rd (bit 7 of IR[8:0])

T3:If (IR[7] = 1): A ← RF[DST]

T4:RF[DST] ← NOT A, update Z/N flags, SC ← 0 — DST from IR[8:6]

CLCReg-RefClear carry flag (bit 6 of IR[8:0])

T3:If (IR[6] = 1): C ← 0, SC ← 0

STCReg-RefSet carry flag (bit 5 of IR[8:0])

T3:If (IR[5] = 1): C ← 1, SC ← 0

CLZReg-RefClear zero flag (bit 4 of IR[8:0])

T3:If (IR[4] = 1): Z ← 0, SC ← 0

NOPReg-RefNo operation (bit 3 of IR[8:0])

T3:If (IR[3] = 1): SC ← 0 — immediately reset SC, nothing else

SWAP Rd, RsReg-RefExchange Rd ↔ Rs via TR (bit 2 of IR[8:0])

T3:If (IR[2] = 1): TR ← RF[DST] — save Rd in temp register

T4:RF[DST] ← RF[SRC] — Rd gets Rs's value

T5:RF[SRC] ← TR, SC ← 0 — Rs gets original Rd value from TR

DUAD-16 Project — Step-by-Step Status

CE222 · Usman, Reyan, Saim · Deadline April 20

1

Design philosophy & innovation angleDone

- 2-address architecture (vs Mano's 1-address accumulator)

- 8 general-purpose registers R0–R7 (vs Mano's special-purpose)

- Removes accumulator bottleneck — core innovation clearly stated

- Named DUAD-16 (2-address, 16-bit)

2

Word size, memory, registersDone

- 16-bit word size confirmed

- Memory: 4096×16 via register-indirect (justified, stronger than 512)

- Registers: R0–R7 (16-bit each), PC (12-bit), IR (16-bit), TR (16-bit)

- Flags: Z, C, N · Flip-flops: IEN, FGI, FGO, S (start/stop)

3

Instruction format + addressing modesDone

- 16-bit format: [15:11] opcode (5-bit) · [10:9] MODE (2-bit) · [8:6] DST (3-bit) · [5:0] SRC (6-bit)

- 4 modes: Register (00), Immediate (01), Direct (10), Register-Indirect (11)

- DST/SRC role explained clearly (TO destination, USING source)

4

Instruction set with encodingsDone

- 33 instructions across 28 opcode encodings (just completed)

- All 4 completeness categories covered: ALU, Memory, Control, I/O

- Register-reference group added (opcode 11110, one-hot bits)

- I/O group added: INP, OUT, SKI, SKO, ION, IOF

- Opcode 11111 reserved for future extension

5

Microoperations for all instructionsPartial

- Fetch/Decode (T0–T2) defined

- Core ALU ops: ADD, SUB, MUL, AND, OR, XOR, MOV written

- LDI, LDM, STM, JMP, JZ, JNZ, HLT, NOT, INC, DEC, SHL, SHR, CMP written

- Interrupt cycle RT0–RT2 defined

- ⚠ NEW instructions need microops: INP, OUT, SKI, SKO, ION, IOF

- ⚠ Register-reference group: CLA, CMR, CLC, STC, CLZ, NOP, SWAP

6

ALU designNot started

Need: S3 S2 S1 S0 Cin control signal table → 16 operations. Block diagram showing arithmetic unit + logic unit + 4×1 MUX. Two inputs (A from Rd, B from Rs), output back to register file.

7

Control Unit designNot started

Need: 5-to-32 decoder (for 5-bit opcode), sequence counter SC, MODE field routing, DST/SRC → register file selector. Explain how it differs from Mano's 3×8 decoder.

8

Common bus + memory interfaceNot started

Need: 16-bit common bus diagram. Sources: Register File output, Memory (DR), PC. MUX structure. How AR gates addresses onto the bus. Compare to Mano's 7-source bus.

9

I/O designNot started

Need: Program-controlled I/O description (SKI/SKO polling loop). Interrupt cycle using IEN, FGI, FGO flags. Interrupt microops RT0–RT2 (already drafted, needs formatting into report section).

10

Complete computer description + flowchartNot started

Need: Master flowchart (Start → Fetch → Decode → type check → Execute → Interrupt check → repeat). Summary table of ALL microoperations. Matches Lecture 22 style exactly.

11

Write the reportNot started

3000+ words, 12 sections, diagrams hand-drawn or PowerPoint, microops in T-notation, Turnitin <19%, AI index must be *. Sections assigned: Usman (main body), Saim (abstract + bibliography), Reyan (simulator manual).

12

Simulator (optional, +2 marks)Pending

HTML/JS simulator already partially built. Needs: all 33 instructions assembled correctly, register file display, flag display, step/run buttons, help panel with all mnemonics.

DUAD-16 Complete Instruction Set

5-bit opcode space → 32 slots. 28 used, 4 reserved for future expansion.

| # | Opcode (bin) | Mnemonic | Operation | Category |
| --- | --- | --- | --- | --- |
| Arithmetic / Logic (ALU) — Register-to-Register |  |  |  |  |
| 1 | 00000 | ADD Rd, Rs | Rd ← Rd + Rs | ALU |
| 2 | 00001 | SUB Rd, Rs | Rd ← Rd − Rs | ALU |
| 3 | 00010 | MUL Rd, Rs | Rd ← Rd × Rs (lower 16 bits) | ALU |
| 4 | 00011 | AND Rd, Rs | Rd ← Rd AND Rs | ALU |
| 5 | 00100 | OR Rd, Rs | Rd ← Rd OR Rs | ALU |
| 6 | 00101 | XOR Rd, Rs | Rd ← Rd XOR Rs | ALU |
| 7 | 00110 | MOV Rd, Rs | Rd ← Rs | ALU |
| 8 | 00111 | CMP Rd, Rs | Set Z/C/N from Rd − Rs, no write-back | ALU |
| 9 | 01000 | NOT Rd | Rd ← NOT Rd | ALU |
| 10 | 01001 | INC Rd | Rd ← Rd + 1 | ALU |
| 11 | 01010 | DEC Rd | Rd ← Rd − 1 | ALU |
| 12 | 01011 | SHL Rd, #n | Rd ← Rd << n (n from IR[3:0]) | ALU |
| 13 | 01100 | SHR Rd, #n | Rd ← Rd >> n (logical) | ALU |
| Data Transfer — Immediate & Memory |  |  |  |  |
| 14 | 01101 | LDI Rd, #imm | Rd ← IR[5:0] (6-bit zero-extended immediate) | Memory |
| 15 | 01110 | LDM Rd, [Rs] | AR←Rs; DR←M[AR]; Rd←DR (register-indirect load) | Memory |
| 16 | 01111 | STM [Rd], Rs | AR←Rd; M[AR]←Rs (register-indirect store) | Memory |
| Program Control — Branches & Halt |  |  |  |  |
| 17 | 10000 | JMP addr | PC ← IR[8:0] (unconditional) | Control |
| 18 | 10001 | JZ Rd, addr | If Rd = 0: PC ← IR[5:0] | Control |
| 19 | 10010 | JNZ Rd, addr | If Rd ≠ 0: PC ← IR[5:0] | Control |
| 20 | 10011 | HLT | S ← 0 (stop flip-flop) | Control |
| I/O Instructions — Program-Controlled |  |  |  |  |
| 21 | 10100 | INP | R0 ← INPR, FGI ← 0 (8-bit char input into R0) | I/O |
| 22 | 10101 | OUT | OUTR ← R0[7:0], FGO ← 0 (output R0 to device) | I/O |
| 23 | 10110 | SKI | If FGI = 1: PC ← PC + 1 (skip if input ready) | I/O |
| 24 | 10111 | SKO | If FGO = 1: PC ← PC + 1 (skip if output ready) | I/O |
| 25 | 11000 | ION | IEN ← 1 (enable interrupts) | I/O |
| 26 | 11001 | IOF | IEN ← 0 (disable interrupts) | I/O |
| Register-Reference Instructions — opcode 11110, bits [8:0] one-hot |  |  |  |  |
| 27 | 11110 — bit 8 | CLA | R0 ← 0 (clear accumulator convention register) | Reg-Ref |
| 28 | 11110 — bit 7 | CMR Rd | Rd ← NOT Rd, update flags (complement any register) | Reg-Ref |
| 29 | 11110 — bit 6 | CLC | C ← 0 (clear carry flag) | Reg-Ref |
| 30 | 11110 — bit 5 | STC | C ← 1 (set carry flag) | Reg-Ref |
| 31 | 11110 — bit 4 | CLZ | Z ← 0 (clear zero flag) | Reg-Ref |
| 32 | 11110 — bit 3 | NOP | No operation (SC ← 0 immediately) | Reg-Ref |
| 33 | 11110 — bit 2 | SWAP Rd, Rs | Rd ↔ Rs via TR (uses temp register) | Reg-Ref |
| Reserved (opcode 11111) — 4 slots open |  |  |  |  |
| — | 11111 | Reserved for future extensions (multiply-high, divide, etc.) |  | — |