DUAD-16 Processor

A Complete Understanding Guide

CE222 – Computer Organization & Assembly Language

Prepared by: Usman, Reyan, Saim

# 1. What Is This Project, In Simple Words?

In this project, we are designing our own custom CPU (Central Processing Unit) from scratch. A CPU is the brain of a computer. It reads instructions one by one, understands them, and executes them.

We are not building it physically with chips and wires. Instead, we are designing it on paper (as a detailed report) and also building a software simulator that shows how our CPU works step by step in a web browser.

Think of it this way: Intel and AMD design real CPUs. We are doing the same thing but at a simple educational level, and our design must be original and different from the examples taught in class.

# 2. What Have We Already Studied In Class?

Before understanding what makes our design unique, you need to know what was already taught:

## 2.1 The Mano Machine

This is a 16-bit computer described in the textbook Computer Systems Architecture by Morris Mano. Here is how it works:

- It has only ONE working register called the Accumulator (AC). All operations happen through this one register.

- To add two numbers, you first load one number into AC, then add the second number to AC. The result goes back into AC.

- Every instruction either refers to the Accumulator or a memory address. There is no freedom to say "put this value into Register 3".

- It uses a 1-address instruction format, meaning each instruction only names one thing (a memory address), and the other operand is always assumed to be the Accumulator.

- Memory size is 4096 words, each word is 16 bits wide.

## 2.2 The 3-Bit Opcode Processor (Taught in Class)

This is a simpler processor your instructor showed with a simulator. Key points:

- It uses only 3 bits for the opcode (the part of the instruction that says what operation to do). 3 bits means only 8 possible instructions.

- Like the Mano Machine, it uses a single accumulator. All math goes through that one register.

- It is 8-bit, meaning it works with 8-bit numbers.

- Very simple and limited. Used mainly to teach the concept of fetch-decode-execute.

Both of these designs follow the same pattern: one accumulator, instructions that operate on that one register, very limited flexibility. This is called a 1-address architecture.

# 3. What Is Our Design? DUAD-16 Explained Simply

Our CPU is called DUAD-16. DUAD stands for Dual-Address, and 16 means it works with 16-bit numbers.

## 3.1 The Big Idea: 2-Address Instructions

In the Mano Machine and 3-bit processor, every instruction only names one thing. For example:

ADD X   means:  AC = AC + Memory[X]

You cannot choose where the result goes. It always goes into AC.

In DUAD-16, every instruction names TWO things: a destination register and a source register. For example:

ADD R2, R5   means:  R2 = R2 + R5

This gives much more freedom. You can add any two registers together and store the result in any register you choose. This is how modern processors actually work (like the x86 or ARM processors in your phone and computer).

## 3.2 Eight General-Purpose Registers

Instead of one accumulator, DUAD-16 has eight registers named R0 through R7. Each register holds a 16-bit value.

Think of registers as small, fast storage boxes inside the CPU itself. Memory is like a large warehouse far away. Registers are like the 8 boxes on your desk that you can reach instantly.

| Register | Name | Common Use |
| --- | --- | --- |
| R0 | General Purpose | Storing results, loop counters |
| R1 | General Purpose | Second operand in calculations |
| R2 | General Purpose | Temporary values |
| R3 | General Purpose | Storing final answers |
| R4 | General Purpose | Loop variables |
| R5 | General Purpose | Intermediate calculations |
| R6 | General Purpose | Address pointer for memory |
| R7 | General Purpose | Stack pointer (advanced use) |

All 8 registers can be used for any purpose. There is no restriction like 'only R0 can do arithmetic'. This is what General Purpose means.

# 4. Memory Organization

## 4.1 How Much Memory?

DUAD-16 has 512 words of memory. Each word is 16 bits (2 bytes). So total memory = 512 x 2 = 1024 bytes = 1 KB.

The memory address is 9 bits wide (because 2^9 = 512). This means you can address 512 different locations, from address 0 all the way to address 511.

| Property | Value | Explanation |
| --- | --- | --- |
| Word size | 16 bits | Each memory cell holds a 16-bit number |
| Number of words | 512 | 512 individual storage cells |
| Address size | 9 bits | 9 bits needed to name any of 512 locations |
| Total storage | 1 KB | 512 x 2 bytes = 1024 bytes |

## 4.2 How Memory Is Used

Memory is divided into two zones in our design:

- Program zone (addresses 0 to ~255): This is where the CPU's instructions are stored. When you write an assembly program and assemble it, the binary instructions are loaded here.

- Data zone (addresses 256 to 511): This is where your program stores and reads variable values during execution.

When the CPU starts running, it begins reading instructions from address 0 and goes upward, one instruction per clock cycle (in our simplified model).

# 5. The Instruction Format In Detail

This is the most important and unique part of our design. Every single instruction in DUAD-16 is exactly 16 bits wide. These 16 bits are divided into 4 fields:

| Field | Bits | Size | Purpose |
| --- | --- | --- | --- |
| OPCODE | Bits 15 to 11 | 5 bits | Says what operation to perform (ADD, SUB, JMP, etc.) |
| MODE | Bits 10 to 9 | 2 bits | Says how to interpret the source operand |
| DST (Destination) | Bits 8 to 6 | 3 bits | Which register gets the result (R0 to R7) |
| SRC (Source) | Bits 5 to 0 | 6 bits | The second operand: a register number or an immediate value |

## 5.1 Worked Example

Let us trace the instruction ADD R2, R5 through the format:

- Operation is ADD, whose opcode is 00001 in binary.

- Mode is REG (register to register), which is 00 in binary.

- Destination is R2. Register 2 in 3 bits is 010.

- Source is R5. Register 5 in 6 bits is 000101.

So the full 16-bit instruction is:

00001  00  010  000101

ADD  REG  R2    R5

Combining all 16 bits: 0000100010000101 in binary, which is 0x0885 in hexadecimal. This one 16-bit number completely describes the instruction.

## 5.2 Why 5-Bit Opcode?

5 bits can represent 2^5 = 32 different values. This means we can have up to 32 different instructions. The Mano Machine uses 3 bits (8 instructions). The 3-bit opcode processor also uses 3 bits. Our 5-bit opcode gives us room for a much richer instruction set with 20 instructions designed so far.

## 5.3 The Four Addressing Modes (2-bit MODE field)

The MODE field tells the CPU how to read the source operand. DUAD-16 supports four modes:

| Mode | Binary Code | Meaning | Example |
| --- | --- | --- | --- |
| Register | 00 | Source is a register R0-R7 | ADD R2, R5 (add R5 into R2) |
| Immediate | 01 | Source is a number written directly in the instruction | LDI R0, #10 (load number 10 into R0) |
| Direct | 10 | Source is a memory address, go read that address | LDM R3, [100] (read memory cell 100 into R3) |
| Register Indirect | 11 | Source is an address stored inside a register | LDM R3, [R6] (R6 holds the address, read that cell) |

Having four addressing modes is one of the key requirements in the project rubric and one area where DUAD-16 clearly surpasses both the Mano Machine and the 3-bit processor.

# 6. Complete Instruction Set

DUAD-16 has 20 instructions covering all five required categories from the project rubric: arithmetic, logic, shift, data movement, and program control.

| Instruction | Opcode (5-bit) | Operation | Category |
| --- | --- | --- | --- |
| ADD Rd, Rs | 00001 | Rd = Rd + Rs | Arithmetic |
| SUB Rd, Rs | 00010 | Rd = Rd - Rs | Arithmetic |
| MUL Rd, Rs | 00011 | Rd = Rd x Rs (lower 16 bits) | Arithmetic |
| INC Rd | 10010 | Rd = Rd + 1 | Arithmetic |
| DEC Rd | 10011 | Rd = Rd - 1 | Arithmetic |
| AND Rd, Rs | 00100 | Rd = Rd AND Rs (bitwise) | Logic |
| OR Rd, Rs | 00101 | Rd = Rd OR Rs (bitwise) | Logic |
| XOR Rd, Rs | 00110 | Rd = Rd XOR Rs (bitwise) | Logic |
| NOT Rd | 00111 | Rd = complement of Rd (flip all bits) | Logic |
| CMP Rd, Rs | 10001 | Compare Rd and Rs, set flags only | Logic |
| SHL Rd, #n | 01000 | Rd = Rd shifted left by n positions | Shift |
| SHR Rd, #n | 01001 | Rd = Rd shifted right by n positions | Shift |
| MOV Rd, Rs | 01010 | Rd = Rs (copy register to register) | Data Move |
| LDI Rd, #imm | 01011 | Rd = immediate number (load constant) | Data Move |
| LDM Rd, [Rs] | 01100 | Rd = Memory[Rs] (load from memory) | Data Move |
| STM [Rd], Rs | 01101 | Memory[Rd] = Rs (store into memory) | Data Move |
| JMP addr | 01110 | PC = address (jump unconditionally) | Control |
| JZ Rd, addr | 01111 | If Rd = 0, jump to address | Control |
| JNZ Rd, addr | 10000 | If Rd not 0, jump to address | Control |
| HLT | 11111 | Stop the CPU | Control |

# 7. CPU Internal Components

## 7.1 The ALU (Arithmetic and Logic Unit)

The ALU is the part of the CPU that actually does the math and logic operations. Every time you execute ADD, SUB, AND, OR, SHL, etc., the ALU performs that operation.

In DUAD-16, the ALU receives two 16-bit inputs (from the register file), performs the operation selected by the Control Unit, and produces a 16-bit output that is written back into the destination register.

The ALU also sets three flags after each operation:

- Zero Flag (Z): Set to 1 if the result is exactly zero. Used by JZ instruction to decide whether to jump.

- Carry Flag (C): Set to 1 if there was a carry out of bit 15 during addition, or a borrow during subtraction.

- Negative Flag (N): Set to 1 if bit 15 of the result is 1, meaning the result is negative (in two's complement representation).

## 7.2 The Control Unit (CU)

The Control Unit is the director of the CPU. It does not do any calculations itself. Instead, it reads each instruction, figures out what it means, and sends control signals to all other parts telling them what to do.

For every instruction, the Control Unit goes through three stages:

- FETCH: Read the instruction from memory at the address in the Program Counter. Load it into the Instruction Register (IR).

- DECODE: Break apart the 16-bit instruction into its four fields (OPCODE, MODE, DST, SRC). Understand what needs to happen.

- EXECUTE: Send signals to the ALU, register file, or memory to carry out the operation. Update the Program Counter to point to the next instruction.

## 7.3 The Register File

The register file is a small, fast block of storage containing our 8 registers (R0 to R7). It has two read ports (it can give out two register values at the same time, one for each ALU input) and one write port (it can receive one result and store it back). This is what makes 2-address instructions efficient.

## 7.4 The Program Counter (PC)

The Program Counter is a special register that always holds the address of the next instruction to be fetched. After each instruction executes, the PC automatically increments by 1. Jump instructions (JMP, JZ, JNZ) change the PC to a different address instead of incrementing, which is how loops and conditions work.

## 7.5 The Instruction Register (IR)

When the CPU fetches an instruction from memory, it stores it temporarily in the Instruction Register while the Control Unit decodes it. IR is 16 bits wide since all our instructions are 16 bits.

# 8. The System Bus

The bus is the highway that connects all parts of the computer together. DUAD-16 uses a simple three-bus structure:

- Address Bus (9-bit, one-directional): Carries the memory address from the CPU to memory. 9 bits because we have 512 memory locations and 2^9 = 512.

- Data Bus (16-bit, bidirectional): Carries data both ways. When reading from memory: data flows from memory to CPU. When writing to memory: data flows from CPU to memory.

- Control Bus (multiple lines, one-directional): Carries control signals from the Control Unit to all other components. Examples: MemRead (tells memory the CPU wants to read), MemWrite (tells memory the CPU wants to write), RegWrite (tells the register file to store a new value).

# 9. How One Instruction Executes: Step-by-Step Trace

Let us trace ADD R2, R5 completely from start to finish:

Step 1 - FETCH

- The PC holds the current address, say address 3.

- The address bus carries value 3 to memory.

- Memory sends back the 16-bit word stored at address 3: 0000100010000101.

- This word is loaded into the IR.

- PC increments to 4.

Step 2 - DECODE

- The Control Unit reads IR = 0000100010000101.

- It extracts OPCODE = 00001 (ADD), MODE = 00 (Register), DST = 010 (R2), SRC = 000101 (R5).

- It prepares control signals: tell register file to output R2 and R5, tell ALU to perform addition, tell register file to write result back into R2.

Step 3 - EXECUTE

- Register file sends value of R2 (say 0008 hex = 8 decimal) and R5 (say 0005 hex = 5 decimal) to the ALU.

- ALU adds them: 8 + 5 = 13.

- Result 13 (000D hex) goes back to the register file and is stored in R2.

- ALU checks: result is not zero, so Z = 0. No carry, so C = 0. Bit 15 is 0, so N = 0.

- R2 now holds 13. The CPU is ready for the next instruction at PC = 4.

# 10. How DUAD-16 Is Different From Everything Else

Here is a direct comparison against what was studied in class:

| Feature | Mano Machine | 3-bit Opcode | DUAD-16 |
| --- | --- | --- | --- |
| Instruction format | 1-address | 1-address | 2-address (completely different) |
| Opcode width | 3 bits | 3 bits | 5 bits (32 possible instructions) |
| Registers | AC only (1 register) | AC only (1 register) | R0 to R7 (8 registers) |
| Word size | 16-bit | 8-bit | 16-bit |
| Memory size | 4096 words | 8 locations | 512 words |
| Addressing modes | Direct, Indirect | Register only | Register, Immediate, Direct, Register-Indirect |
| ADD operation | AC = AC + Mem[X] | AC = AC + Rx | ADD Rd, Rs (free choice of registers) |
| Number of instructions | 25 approx. | 8 maximum | 20 instructions |
| Flags | E, C | Z only | Z, C, N (three flags) |
| Real-world similarity | 1950s style | Teaching toy | Closer to modern RISC processors |

The single biggest difference is the instruction format. DUAD-16 is a 2-address machine. Neither the Mano Machine nor the 3-bit opcode processor is a 2-address machine. This is not a small change. It fundamentally changes how programs are written, how the register file is built, how the ALU receives its inputs, and how the datapath (internal wiring of the CPU) is organized.

# 11. The Web Simulator

Along with the report, we are building a web-based simulator that runs in any browser. The simulator is a single HTML file that requires no installation.

## 11.1 What the Simulator Does

- You type DUAD-16 assembly code in a text editor on the left.

- You click Assemble and the simulator converts your text into 16-bit binary machine code.

- You click Step to execute one instruction at a time, or Run to execute automatically at a speed you choose.

- You can watch every register (R0 to R7) update live as each instruction runs.

- You can see the Zero, Carry, and Negative flags change.

- You can see the Program Counter move forward and jump when branch instructions execute.

- The memory view shows the first 32 memory cells with the current address highlighted.

- An execution log at the bottom shows every instruction that has run, with the result.

## 11.2 Sample Program That Runs In The Simulator

This program sums the numbers 1 through 5 and stores the result in R3:

; Sum of 1 to 5

LDI R0, #5      ; R0 = 5  (counter)

LDI R1, #0      ; R1 = 0  (running sum)

LDI R2, #1      ; R2 = 1  (constant, used for decrement)

LOOP:

ADD R1, R0      ; sum = sum + counter

SUB R0, R2      ; counter = counter - 1

JNZ R0, LOOP    ; if counter is not zero, go back to LOOP

MOV R3, R1      ; store final answer in R3

HLT             ; stop

After this program runs, R3 holds 15 (which is 1+2+3+4+5 = 15). This can be verified live in the simulator.

## 11.3 Why the Simulator Earns Extra Marks

The project rubric gives 2 absolute bonus marks for a simulator that shows fetch, decode, and execute. Our simulator does exactly this. The stage label (FETCH / DECODE / EXECUTE) updates visually at each step, and the execution log shows exactly what operation ran and what the result was.

# 12. Report Structure and Team Responsibilities

| Section | Who Writes It | Content |
| --- | --- | --- |
| Front Cover | All together | Project title, institute name, course, section, group members with registration numbers, date |
| Turnitin Reports | Librarian stamps | Similarity and AI index reports, signed by librarian |
| Abstract | Saim | Short summary of the novelty of DUAD-16 design |
| Theory & Design (3000+ words) | Usman (main) | Instruction set, ALU design, CU design, instruction formats, addressing modes, memory organization, complete computer description with diagrams |
| Simulator Manual | Saim | How to open and use the simulator, what each button does, example programs |
| Bibliography | Saim | List of all references: Mano textbook, any other sources |

# 13. Four-Week Timeline

| Week | What Each Person Does |
| --- | --- |
| Week 1 (Now) | All: finalize and agree on the instruction set table. Usman: draw the ALU block diagram and CU micro-operations table. Reyan: build the HTML page skeleton with all panels. Saim: write the assembler for at least 5 instructions as a working proof. |
| Week 2 | Usman: complete instruction format section and addressing modes section for the report. Reyan: get register panel and memory view working live. Saim: complete the full fetch-decode-execute loop in JavaScript for all 20 instructions. |
| Week 3 | All: connect Reyan's frontend to Saim's engine. Test every instruction, fix bugs. Usman: write the full Theory & Design section (3000+ words minimum). Run through sample programs on the simulator. |
| Week 4 | Polish the simulator visuals. Saim writes the simulator manual. Complete the report (abstract, front cover, bibliography). Take the report to the librarian for Turnitin. Print, bind, and submit. Practice the presentation together. |

# 14. What the Final Submission Looks Like

Report: A printed, soft-bound A4 document containing: front cover, Turnitin reports (librarian-stamped), abstract, Theory & Design section (minimum 3000 words, with tables and block diagrams), simulator manual, and bibliography.

Simulator file: A single HTML file named DUAD16_Simulator.html that opens in any browser and runs the complete DUAD-16 simulation with all 20 instructions.

Presentation: All three group members present the design in class. Each member explains their part. The instructor will ask questions. Knowing your own section deeply is essential.

# 15. Key Numbers to Remember

| Property | Value |
| --- | --- |
| Word size | 16 bits |
| Number of registers | 8 (R0 to R7) |
| Memory words | 512 |
| Memory address width | 9 bits |
| Instruction width | 16 bits (fixed) |
| Opcode field | 5 bits (bits 15-11) |
| Mode field | 2 bits (bits 10-9) |
| Destination register field | 3 bits (bits 8-6) |
| Source field | 6 bits (bits 5-0) |
| Number of addressing modes | 4 |
| Total instructions | 20 |
| Flags | 3 (Zero, Carry, Negative) |
| Data bus width | 16 bits |
| Address bus width | 9 bits |

End of Understanding Document

CE222 Semester Project – DUAD-16 Processor – Usman, Reyan, Saim